#include <Wire.h>

#include <Adafruit_VL53L0X.h>

#include <Servo.h>

#include <PID_v1.h>

// ═══════════════════════════════════════════════════

//  PINS

// ═══════════════════════════════════════════════════

#define SERVO_PIN       9

#define POT_PIN         A3

#define POT_POWER_PIN   12

// ═══════════════════════════════════════════════════

//  SERVO CONFIGURATION  ◄─ TUNE THESE FIRST

// ═══════════════════════════════════════════════════

#define SERVO_CENTER    90    // Angle when beam is perfectly level

#define SERVO_MIN       60    // Max tilt toward sensor end

#define SERVO_MAX       120   // Max tilt away from sensor

// ═══════════════════════════════════════════════════

//  SENSOR / BEAM CONFIGURATION

// ═══════════════════════════════════════════════════

#define SENSOR_MIN_MM   20    // 2 cm = 20 mm

#define SENSOR_MAX_MM   390   // 39 cm = 390 mm

// ═══════════════════════════════════════════════════

//  MOVING AVERAGE FILTER

// ═══════════════════════════════════════════════════

#define NUM_SAMPLES     8

// ═══════════════════════════════════════════════════

//  UPDATE RATES

// ═══════════════════════════════════════════════════

#define UPDATE_INTERVAL   20    // ms — control loop (50 Hz)

#define DIAG_INTERVAL    500    // ms — diagnostic report rate

// ═══════════════════════════════════════════════════

//  PID TUNING  ◄─ CHANGE THESE TO TUNE

// ═══════════════════════════════════════════════════

double Kp = 2.5;

double Ki = 0.05;

double Kd = 0.08;

// ═══════════════════════════════════════════════════

//  OBJECTS & STATE

// ═══════════════════════════════════════════════════

Adafruit_VL53L0X tof;

Servo             servo;

double pidInput, pidOutput, targetDist;

PID pid(&pidInput, &pidOutput, &targetDist, Kp, Ki, Kd, DIRECT);

double        samples[NUM_SAMPLES];

int           sampleIndex   = 0;

double        averageDist   = (SENSOR_MIN_MM + SENSOR_MAX_MM) / 2.0;

unsigned long previousMillis     = 0;

unsigned long previousDiagMillis = 0;

// ── Feedback / diagnostics state ─────────────────

bool   plotterMode   = true;

double errorNow      = 0;

double prevError     = 0;

double peakError     = 0;

double errorSum      = 0;

int    oscillCount   = 0;

bool   lastErrorPos  = true;

int    settledCycles = 0;

bool   isSettled     = false;

unsigned long lastCrossTime = 0;

unsigned long oscillPeriod  = 0;

#define SETTLE_DEADBAND_MM  5

#define SETTLE_CYCLES_REQ   25

#define MAX_HISTORY         20

double errorHistory[MAX_HISTORY];

int    historyIdx = 0;

// ═══════════════════════════════════════════════════

//  HELPER — MOVING AVERAGE

// ═══════════════════════════════════════════════════

double movingAverage(double newVal) {

samples[sampleIndex] = newVal;

sampleIndex = (sampleIndex + 1) % NUM_SAMPLES;

double sum = 0;

for (int i = 0; i < NUM_SAMPLES; i++) sum += samples[i];

return sum / NUM_SAMPLES;

}

// ═══════════════════════════════════════════════════

//  HELPER — SAFE SERVO WRITE

// ═══════════════════════════════════════════════════

void writeServo(double angle) {

int a = constrain((int)round(angle), SERVO_MIN, SERVO_MAX);

servo.write(a);

}

// ═══════════════════════════════════════════════════

//  UPDATE DIAGNOSTIC STATE — called every control cycle

// ═══════════════════════════════════════════════════

void updateDiagnostics() {

errorNow = targetDist - averageDist;

errorSum += errorNow;

if (abs(errorNow) > abs(peakError)) peakError = errorNow;

// Count zero-crossings for oscillation detection

bool posNow = (errorNow >= 0);

if (posNow != lastErrorPos) {

oscillCount++;

unsigned long now = millis();

if (lastCrossTime > 0) oscillPeriod = now - lastCrossTime;

lastCrossTime = now;

lastErrorPos  = posNow;

}

// Settled detection

if (abs(errorNow) < SETTLE_DEADBAND_MM) {

settledCycles++;

if (settledCycles >= SETTLE_CYCLES_REQ) isSettled = true;

} else {

settledCycles = 0;

isSettled     = false;

}

// Rolling error history for noise analysis

errorHistory[historyIdx] = errorNow;

historyIdx = (historyIdx + 1) % MAX_HISTORY;

prevError = errorNow;

}

// ═══════════════════════════════════════════════════

//  PLOTTER OUTPUT — open Tools > Serial Plotter

// ═══════════════════════════════════════════════════

void printPlotter(double servoAngle) {

Serial.print("Distance:");  Serial.print(averageDist);  Serial.print(",");

Serial.print("Target:");    Serial.print(targetDist);   Serial.print(",");

Serial.print("Error:");     Serial.print(errorNow);     Serial.print(",");

Serial.print("Servo:");     Serial.print(servoAngle);   Serial.print(",");

Serial.print("PID_Out:");   Serial.println(pidOutput);

}

// ═══════════════════════════════════════════════════

//  DIAGNOSTIC REPORT — open Tools > Serial Monitor

// ═══════════════════════════════════════════════════

void printDiagnostic(double servoAngle) {

// Noise measurement — std deviation of recent errors

double mean = 0;

for (int i = 0; i < MAX_HISTORY; i++) mean += errorHistory[i];

mean /= MAX_HISTORY;

double variance = 0;

for (int i = 0; i < MAX_HISTORY; i++) {

double d = errorHistory[i] - mean;

variance += d * d;

}

double stdDev = sqrt(variance / MAX_HISTORY);

Serial.println(F("══════════════════════════════════════"));

Serial.println(F("   BALL ON BEAM — LIVE DIAGNOSTIC"));

Serial.println(F("══════════════════════════════════════"));

// ── Live values ──────────────────────────────

Serial.println(F("\n[ LIVE VALUES ]"));

Serial.print(F("  Ball position  : ")); Serial.print(averageDist, 1); Serial.println(F(" mm"));

Serial.print(F("  Target         : ")); Serial.print(targetDist,  1); Serial.println(F(" mm"));

Serial.print(F("  Error          : ")); Serial.print(errorNow,    1); Serial.println(F(" mm"));

Serial.print(F("  Servo angle    : ")); Serial.print(servoAngle,  1); Serial.println(F(" deg"));

Serial.print(F("  PID output     : ")); Serial.println(pidOutput, 2);

// ── Current gains ────────────────────────────

Serial.println(F("\n[ CURRENT PID GAINS ]"));

Serial.print(F("  Kp = ")); Serial.print(Kp, 5);

Serial.print(F("   Ki = ")); Serial.print(Ki, 6);

Serial.print(F("   Kd = ")); Serial.println(Kd, 5);

// ── Behaviour stats ──────────────────────────

Serial.println(F("\n[ BEHAVIOUR ANALYSIS ]"));

Serial.print(F("  Peak error seen    : ")); Serial.print(abs(peakError), 1); Serial.println(F(" mm"));

Serial.print(F("  Sensor noise (std) : ")); Serial.print(stdDev, 1);         Serial.println(F(" mm"));

Serial.print(F("  Oscillations       : ")); Serial.print(oscillCount);       Serial.println(F(" zero-crossings"));

if (oscillPeriod > 0) {

Serial.print(F("  Oscillation period : ")); Serial.print(oscillPeriod); Serial.println(F(" ms"));

}

if (isSettled) {

Serial.println(F("  Status             : SETTLED (error < 5 mm)"));

} else if (abs(errorNow) < SETTLE_DEADBAND_MM) {

Serial.print(F("  Status             : Converging ("));

Serial.print(settledCycles); Serial.print(F("/"));

Serial.print(SETTLE_CYCLES_REQ); Serial.println(F(" cycles)"));

} else {

Serial.println(F("  Status             : NOT settled"));

}

// ── Tuning advice ─────────────────────────────

Serial.println(F("\n[ TUNING ADVICE ]"));

bool adviceGiven = false;

// Wrong direction

if (errorNow > 80 && pidOutput < -10) {

Serial.println(F("  >> Ball moves WRONG WAY"));

Serial.println(F("     Fix: change DIRECT to REVERSE in PID constructor"));

adviceGiven = true;

}

// Fast oscillation — Kd too low

if (oscillCount > 8 && !isSettled && oscillPeriod > 0 && oscillPeriod < 500) {

Serial.println(F("  >> FAST oscillation detected (period < 500ms)"));

Serial.println(F("     Fix: increase Kd (try doubling it)"));

Serial.println(F("          also try reducing Kp by 20%"));

adviceGiven = true;

}

// Slow oscillation — Kp too high

else if (oscillCount > 8 && !isSettled && oscillPeriod >= 500) {

Serial.println(F("  >> SLOW oscillation detected (period > 500ms)"));

Serial.println(F("     Fix: increase Kd first"));

Serial.println(F("          if already high, reduce Kp by 20%"));

adviceGiven = true;

}

// Settled but offset from target — needs Ki

if (isSettled && abs(errorNow) > SETTLE_DEADBAND_MM * 1.5) {

Serial.println(F("  >> Settled but NOT at target (steady-state error)"));

Serial.println(F("     Fix: increase Ki (try Ki * 2)"));

adviceGiven = true;

}

// Sluggish — not moving enough

if (!isSettled && abs(errorNow) > 60 && abs(pidOutput) < 4) {

Serial.println(F("  >> Ball barely moving toward target (sluggish)"));

Serial.println(F("     Fix: increase Kp (try Kp * 1.5)"));

adviceGiven = true;

}

// Servo at limits

if (servoAngle <= SERVO_MIN + 2 || servoAngle >= SERVO_MAX - 2) {

Serial.println(F("  >> Servo hitting mechanical limits"));

Serial.println(F("     Fix: reduce Kp, or widen SERVO_MIN/SERVO_MAX if safe"));

adviceGiven = true;

}

// High sensor noise

if (stdDev > 15.0) {

Serial.println(F("  >> High sensor noise (std-dev > 15mm)"));

Serial.println(F("     Fix: increase NUM_SAMPLES to 12 or 14"));

adviceGiven = true;

}

// Integral wind-up

if (abs(errorSum) > 80000) {

Serial.println(F("  >> Integral wind-up risk (errorSum very large)"));

Serial.println(F("     Fix: reduce Ki, or type 'r' to reset stats"));

adviceGiven = true;

}

if (!adviceGiven && isSettled) {

Serial.println(F("  >> System looks well-tuned. Ball is settled at target."));

Serial.println(F("     Try moving the pot to a new target to stress-test it."));

} else if (!adviceGiven) {

Serial.println(F("  >> Still converging — watch for oscillation or sluggishness."));

}

Serial.println(F("\n  [ 'p' = plotter | 'd' = diagnostic | 'r' = reset stats ]"));

Serial.println();

}

// ═══════════════════════════════════════════════════

//  SETUP

// ═══════════════════════════════════════════════════

void setup() {

Serial.begin(115200);

pinMode(POT_POWER_PIN, OUTPUT);

digitalWrite(POT_POWER_PIN, HIGH);

// ── VL53L0X ───────────────────────────────────

Wire.begin();

if (!tof.begin()) {

Serial.println(F("ERROR: VL53L0X not found! Check SDA(A4) SCL(A5) and 3.3V."));

while (1);

}

tof.configSensor(Adafruit_VL53L0X::VL53L0X_SENSE_LONG_RANGE);

// ── Servo ─────────────────────────────────────

servo.attach(SERVO_PIN);

writeServo(SERVO_CENTER);

delay(600);

// ── Pre-fill arrays ───────────────────────────

double mid = (SENSOR_MIN_MM + SENSOR_MAX_MM) / 2.0;

for (int i = 0; i < NUM_SAMPLES;  i++) samples[i]      = mid;

for (int i = 0; i < MAX_HISTORY;  i++) errorHistory[i] = 0;

averageDist = mid;

// ── PID ───────────────────────────────────────

pid.SetMode(AUTOMATIC);

pid.SetSampleTime(UPDATE_INTERVAL);

pid.SetOutputLimits(-(SERVO_CENTER - SERVO_MIN),

(SERVO_MAX   - SERVO_CENTER));

// ── Startup message ───────────────────────────

Serial.println(F(""));

Serial.println(F("╔══════════════════════════════════════╗"));

Serial.println(F("║     BALL ON BEAM — READY             ║"));

Serial.println(F("╠══════════════════════════════════════╣"));

Serial.println(F("║  'p'  Open Tools > Serial Plotter    ║"));

Serial.println(F("║  'd'  Open Tools > Serial Monitor    ║"));

Serial.println(F("║  'r'  Reset statistics               ║"));

Serial.println(F("╚══════════════════════════════════════╝"));

Serial.println(F(""));

Serial.println(F("Starting in PLOTTER mode."));

}

// ═══════════════════════════════════════════════════

//  MAIN LOOP

// ═══════════════════════════════════════════════════

void loop() {

unsigned long now = millis();

// ── Handle serial commands ────────────────────

if (Serial.available()) {

char cmd = Serial.read();

if      (cmd == 'p' || cmd == 'P') { plotterMode = true;  Serial.println(F(">> PLOTTER MODE — open Tools > Serial Plotter")); }

else if (cmd == 'd' || cmd == 'D') { plotterMode = false; Serial.println(F(">> DIAGNOSTIC MODE")); }

else if (cmd == 'r' || cmd == 'R') {

peakError = 0; errorSum = 0; oscillCount = 0;

settledCycles = 0; isSettled = false; oscillPeriod = 0;

Serial.println(F(">> Statistics reset."));

}

}

// ── Control loop (50 Hz) ──────────────────────

if (now - previousMillis >= UPDATE_INTERVAL) {

previousMillis = now;

// 1. Read sensor

VL53L0X_RangingMeasurementData_t measure;

tof.rangingTest(&measure, false);

double rawDist;

if (measure.RangeStatus != 4) {

rawDist = (double)measure.RangeMilliMeter;

} else {

rawDist = averageDist;

}

rawDist = constrain(rawDist, SENSOR_MIN_MM, SENSOR_MAX_MM);

// 2. Filter

averageDist = movingAverage(rawDist);

// 3. Target from pot

targetDist = map(analogRead(POT_PIN), 0, 1023, SENSOR_MIN_MM, SENSOR_MAX_MM);

// 4. PID compute

pidInput = averageDist;

pid.Compute();

// 5. Drive servo

double servoAngle = SERVO_CENTER + pidOutput;

writeServo(servoAngle);

// 6. Update diagnostic state

updateDiagnostics();

// 7. Plotter output (every cycle for smooth graph)

if (plotterMode) printPlotter(servoAngle);

}

// ── Diagnostic report (every 500 ms) ─────────

if (!plotterMode && (now - previousDiagMillis >= DIAG_INTERVAL)) {

previousDiagMillis = now;

printDiagnostic(SERVO_CENTER + pidOutput);

}

}